| Description | OQ Alarm: MX_955_MD_TargetGoodReached |  |  |  |
| --- | --- | --- | --- | --- |
|  | Preset number of pieces for good tubes has been reached |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Press button \Statistics\Production\Resetting counter (s) CHECK PATH Set operation mode \Basic settings\Settings\Target count to 5 CHECK PATH Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops after 5 detected good packages - (Depending on machine configuration more tubes will be ejected, counting of tubes happens at the last eject station) |  |  |  |
|  | The following message appears: |  |  |  |
|  | Preset number of pieces for good tubes has been reached |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set „Target piece“ to 0 on HMI” Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |