| Description | OQ Alarm: MX_456_MF_VacMonitor |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Vacuum low on suction cup @1s@. Please remove product. Then confirm removal here. |  |  |  |
| Prerequisites | - Machine is ready for operation and filled with product |  |  |  |
| Step 1 | - Press the START-button - Wait until tubes have been picked up by the robot. - Press the stop button. - Remove one product from any of the suction cups. - Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine stops |  |  |  |
|  | - The following message appears: (@1s@ will be replaced at the HMI with the actual number from the suction cup were the product was removed) |  |  |  |
|  | Vacuum low on suction cup @1@. Please remove product. Then confirm removal here. |  |  |  |
| Step 2 | - Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - Press the RESET-button at the dialog message at the HMI |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared |  |  |  |
|  | - Machine status: ”Ready for operation” |  |  |  |