| Description | OQ Alarm: MX_401_MS_MinSupply |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min. supply, product, fallen below |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Go to \ Product hopper - monitoring, fill level \ Monitoring, fill level check path Set value of “Target fill level value for min. supply” to a value upper than actual value “Fill level [%] “(Product hopper). Transmit settings to the control system Warning message appears on the screen |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The following message appears: |  |  |  |
|  | Min. supply, product, fallen below |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set value of “Target fill level value for min. supply” to the value noted in step 1 check text. Transmit settings to the control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |