| Description | OQ Alarm: MX_401_MS_Overfill |  |  |  |
| --- | --- | --- | --- | --- |
|  | Overfill prevention is activated |  |  |  |
| Prerequisites | Machine is running and produces tubes |  |  |  |
| Step 1 | Disconnect sensor .401-B2 (Check BMK) |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Overfill prevention is activated |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Reinstallation sensor .401-B2 Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |