| Description | OQ Alarms: MX_255_MS_ConsFault3 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Tube code, station 3 is faulty |  |  |  |
| Prerequisites | Machine is ready for operation. Code reading is activated Manipulate code of 3 tubes, so the code is different to the set one and mark them; then insert these tubes in sequence into track 3 Note value of /Monitoring, tube code/Monitoring, tube code A “Consecutive fault”: ____ Check Path Set “Consecutive fault” to 3 Check Path Transmit settings to the controller |  |  |  |
| Step 1 | Press the START-button. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after the 3 manipulated tubes passed the code reader |  |  |  |
|  | Following message appears: |  |  |  |
|  | Consecutive fault: Tube code, station 3 is faulty |  |  |  |
| Step 2 | Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault” back to noted value Transmit settings to the controller |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Error message disappears. |  |  |  |
|  | Machine status: Ready for Operation |  |  |  |