| Description | OQ Alarm: MX_256_MS_CfCapPres2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Consecutive fault: Tube cap station 2 is faulty |  |  |  |
| Prerequisites | - Machine is ready for operation |  |  |  |
| Step 1 | - Activate tube cap monitoring at the HMI - Note value “Monitoring, tube cap presence”, “Consecutive fault(s)” _________ check Path - Set “Consecutive fault(s)” to 2 - Insert 2 tubes without caps into tube holders for station 2 - Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Machine stops after the second tube has passed the tube cap monitoring station |  |  |  |
|  | - The following message appears: |  |  |  |
|  | Consecutive fault: Tube cap station 2 is faulty |  |  |  |
| Step 2 | - Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - • Set “Consecutive fault(s)” to the value noted in step 1 - Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared |  |  |  |
|  | - Machine status: ”Ready for operation” |  |  |  |