| Description | OQ Alarm: MX_270_MS_Vacuum |  |  |  |
| --- | --- | --- | --- | --- |
|  | Vakuumüberwachung Tubenausblasung hat ausgelöst |  |  |  |
| Prerequisites | Machine is ready for operation / Operation/ Tube blow out/ Settings/ Tube blow out is switched to ON CHECK PATH |  |  |  |
| Step 1 | Disconnect wire at input .004-K20.4:LED 4 CHECK INPUT Press the START-button Activate tube infeed and wait until tubes have passed the tube blow out. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Vakuumüberwachung Tubenausblasung hat ausgelöst |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Reconnect wire Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |