| Description | OQ Alarm: MX_321_MS_Tubeholder3 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Tube holder is not in carrier at filling station 3 |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Remove a tube holder for track 3 before sensor .321-B2 (Check BMK) Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Tube holder is not in carrier at filling station 3 |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Re-insert the tube holder Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |