| Description | OQ Alarm: MX_401_MS_MIinSupply |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min. supply, product, fallen below |  |  |  |
| Prerequisites | Machine is ready for operation Hopper fill level is above Target fill level value, pump off Monitoring, hopper fill level is set to Automatic |  |  |  |
| Step 1 | Go to \Operation\Product hopper\Monitoring, fill level check path Note value of Target fill level for min. supply: ___________ check text Note value of Target fill level for pump on: ___________ check text Set value of Fill level recording, min. backup to a value above Hopper fill level, if necessary increase value of Fill level value, pump on Transmit settings to the control system Press the START-Button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Min. supply, product, fallen below |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set value of Target fill level for min. supply to the value noted in step 1 check text Set value of Target fill level for pump on to the value noted in step 1 check text Transmit settings to the control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |