| Description | OQ Alarm: MX_005_MS_CompressedAirLow |  |  |  |
| --- | --- | --- | --- | --- |
|  | Compressed air low |  |  |  |
| Prerequisites | Machine is running without product. |  |  |  |
| Step 1 | Register current pressure at service unit: ___ bar Reduce pressure under 4 bar. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops immediately |  |  |  |
|  | The following message appears: |  |  |  |
|  | Compressed air low |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |  |
|  | The Machine cannot be started. |  |  |  |  |
| Step 3 | Set pressure at the service unit to the registered value. Press the RESET-button |  |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |  |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |