| Description | OQ Alarm: MX_006_MS_FunctionRequired |  |  |  |
| --- | --- | --- | --- | --- |
|  | In operation not all the functions necessary for the format are switched on |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Select \Operation\Production check path Select Upstream machine\Upstream machine check path Note the state of „Required in format“: ________ Activate „Required in format“, if it is disabled Select Upstream machine\Settings check path Note the state of Upstream machine: _______ check path Switch OFF Upstream machine check path Transmit parameters Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine does not start |  |  |  |
|  | The following message appears: |  |  |  |
|  | In operation not all the functions necessary for the format are switched on |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Select Upstream machine\Upstream machine check path Set „Required in format“ to the state noted in step 1 Select Upstream machine\Settings check path Set the state of Upstream machine to the state noted in step 1 check path Transmit parameters Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |