| Description | OQ Alarm: MX_051_MS_SFCartonMissingFront |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Carton missing in carton chain for gluing, front |  |  |  |
| Prerequisites | Machine is ready for operation and filled with product |  |  |  |
| Step 1 | Switch \Gluing\Settings\Gluing front to “Auto” Bitte Text prüfen Note the value of\Gluing\Carton sensing\Consecutive fault _____ Bitte TEXT prüfen and set it to „1“ Transmit setting to the control system Press the START-button and wait until the first carton is in carton chain. Take a carton from carton chain Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Carton missing in carton chain for gluing, front |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET button Set “Consecutive fault” back to registered value Transmit setting to the control system |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |