| Description | OQ Alarm: MX_101_MF_CartonMissing |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Carton in carton chain |  |  |  |
| Prerequisites | Machine ready for operation and filled with product. |  |  |  |
| Step 1 | Switch \Carton\Settings\Carton to “Auto” Bitte Text prüfen Note the value of\Carton\Carton erection system\Consecutive fault: _____ Bitte TEXT prüfen and set it to „1“ Transmit setting to the control system Press the START-button and wait until the first carton is drawn off. Press an EMERGENCY STOP-button and take a carton from cycloid. Unlock the EMERGENCY STOP-button and press the RESET-button Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after some cycles in home run position |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Carton in carton chain |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started |  |  |  |
| Step 3 | Remove product assigned to the removed carton from product chain. Press the RESET-button Set “Consecutive fault, carton pickup at counter suction cup” back to registered value Transmit setting to the control system |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |