| Description | OQ Alarm: MX_101_MS_SfCartonMissingCS |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Carton missing in carton chain. Please remove affected cartons. Then confirm removal here. |  |  |  |
| Prerequisites | Machine is ready for operation and filled with product |  |  |  |
| Step 1 | Switch \Carton\Settings\Carton to “Auto” Bitte Text prüfen Note the value of\Carton\Carton cycloid\Consecutive fault, carton, counter suction cup: _____ Bitte TEXT prüfen and set it to „1“ Transmit setting to the control system Press the START-button and wait until the first carton is drawn off. Press an EMERGENCY STOP-button before the carton reaches the counter suction cup and take the carton from cycloid . Unlock the EMERGENCY STOP-button and press the RESET-button Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Carton missing in carton chain. Please remove affected cartons. Then confirm removal here. |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET button at the dialog message in the HMI |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |