| Description | OQ Alarm: MX_102_MF_ProdNotInsert |  |  |  |
| --- | --- | --- | --- | --- |
|  | Product was not completely inserted |  |  |  |
| Prerequisites | Machine is ready for operation and filled up with products |  |  |  |
| Step 1 | Open the protective guarding over the insertion station Pull an already inserted product app. 10 mm out of the carton before it passed light barrier .102-B1 Check CIS Close protective guarding and press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine starts and stops after sensor .102-B1 Bitte BMK prüfen is reached |  |  |  |
|  | The following dialog message appears: |  |  |  |
|  | Product was not completely inserted |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started |  |  |  |
| Step 3 | Re-insert the product Press the RESET button at the dialog message in the HMI |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |