| Description | OQ Alarm: MX_103_MS_ConsFaultFront |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault, dust flap, front is faulty |  |  |  |
| Prerequisites | Machine is ready for operation and filled with products Activate \Monitoring, dust flaps\Settings “Monitoring, dust flaps” CHECK Text Register value of \Monitoring, dust flaps\Monitoring, dust flaps “Consecutive fault(s)”: ___ check text and set it to „3“ Send setting to the control system |  |  |  |
| Step 1 | Manipulate the trailing frontal dust flap of 3 cartons, so that sensor .103-B1 check BMK can detect the flap. Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | After the 3 manipulated cartons have passed the sensor, the machine stops |  |  |  |
|  | Following message appears: |  |  |  |
|  | Consecutive fault, dust flap, front is faulty |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set Consecutive fault(s) to noted value and transmit setting to the control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |