| Description | OQ Alarm: MX_105_MS_SensorFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Dynamic sensor monitoring, leaflet in carton |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Cover Sensor .105-B1 with a leaflet, to activate the sensor permanently CHEK CIS Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine starts and stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Dynamic sensor monitoring, leaflet in carton |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Uncover sensor Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |