| Description | OQ Alarm: MX_105_MS_SfLeafPres |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Leaflet in carton |  |  |  |
| Prerequisites | Machine is ready for operation and filled with products |  |  |  |
| Step 1 | Activate \Monitoring, leaflet in carton\Settings “Monitoring, leaflet in carton” Text prüfen as well as all necessary monitoring and feeding functions. Note the value of \Monitoring, leaflet in carton\ Monitoring, leaflet in carton “Consecutive fault(s)”: ___ Bitte TEXT / Pfad prüfen and set it to „3“ Transmit parameter to the control system Remove three leaflets from the leaflet gripper in sequence and acknowledge the messages. Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine starts and stops in home position. |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Leaflet in carton |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault(s)” to the value noted in step 1 and transmit parameter to the control system |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |