| Description | OQ Alarm: MX_109_MS_SfMonitoring |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault:Monitoring, label presence |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Note the value of \Monitoring, label presence\Consecutive fault, label presence:____ check path Set “Consecutive fault” to 2 and transmit parameter to the control system Move 2 bottles without label in sequence behind sensor .109.B1 check CIS Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault:Monitoring, label presence |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set “Consecutive fault” to the value noted in step 1 and transmit parameter to the control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |