| Description | OQ Alarms: MX_111_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Product completeness in product transport |  |  |  |
| Prerequisites | Machine is ready for operation \Monitoring for completeness\Settings\Monitoring for completeness is activated Bitte TEXT prüfen |  |  |  |
| Step 1 | Manipulate the products three times. Note the value \Monitoring for completeness\Monitoring completeness\Consecutive fault(s): _____ Bitte Pfad /TEXT prüfen Set “Consecutive fault(s)” to 3 and transmit setting to the control system Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after products have passed the camera |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Product completeness in product transport |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault(s)” to the value noted in step 1 and transmit parameter to control system. Bitte TEXT prüfen |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |