| Description | OQ Alarm: MX_114_MF_VerticalProduct |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Upright product in product chain. Please remove product and then confirm removal here. |  |  |  |
| Prerequisites | - Machine is ready for operation |  |  |  |
| Step 1 | - Insert a product upright into the product chain - Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine stops |  |  |  |
|  | - The following message appears: |  |  |  |
|  | Upright product in product chain. Please remove product and then confirm removal here. |  |  |  |
| Step 2 | - Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - Remove the product - Press the RESET-button at the dialog message in the HMI |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared. |  |  |  |
|  | - Machine status: ”Ready for operation” |  |  |  |