| Description | OQ Alarms: MX_117_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Monitoring, tray |  |  |  |
| Prerequisites | Machine is ready for operation \Monitoring, tray\Settings\Monitoring, tray is activated Bitte TEXT prüfen |  |  |  |
| Step 1 | Manipulate the trays three times. Note the value \Monitoring, tray\Monitoring, tray\Consecutive fault: _____ Bitte Pfad /TEXT prüfen Set “Consecutive fault” to 3 and transmit setting to the control system Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after trays have passed the camera |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Monitoring, tray |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault” to the value noted in step 1 and transmit parameter to control system. Bitte TEXT prüfen |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |