| Description | OQ Alarms: MX_153_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault, leaflet code bottom is faulty |  |  |  |
| Prerequisites | Machine is ready for operation \Monitoring, leaflet code\Settings “Monitoring, leaflet code, bottom” on HMI is activated Bitte TEXT prüfen |  |  |  |
| Step 1 | Manipulate the code on the bottom side of 3 leaflets in sequence so it cannot be read and place them in sequence below the leaflet stack. Note the value \Monitoring, leaflet code\Monitoring, leaflet code, bottom “Consecutive fault(s)”: ___ Bitte Pfad /TEXT prüfen Set “Consecutive fault(s)” to 3 and transmit setting to the control system Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops in homerun position after the leaflets with the unreadable code have passed the code scanner |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault, leaflet code bottom is faulty |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault(s)” to the noted value and transmit parameter to control system |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |