| Description | OQ Alarms: MX_154_MS_SfCode |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Booklet code |  |  |  |
| Prerequisites | Machine is ready for operation \Code Add On\Settings\Monitoring, code add on is activated Bitte TEXT prüfen |  |  |  |
| Step 1 | Manipulate the code on 3 booklets so it cannot be read and place the booklets in sequence in front of the booklet magazine. Note the value \ Code Add On \ Code Add On \Consecutive fault(s): _____ Bitte Pfad /TEXT prüfen Set “Consecutive fault(s)” to 3 and transmit setting to the control system Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after the cartons with the unreadable code have passed the code scanner |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Booklet code |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set “Consecutive fault(s)” to the value noted in step 1 and transmit parameter to control system Bitte TEXT prüfen |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |