| Description | OQ Alarms: MX_159_MS_SfCode |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Bottle cap color |  |  |  |
| Prerequisites | Machine is ready for operation \Monitoring, cap color\Settings\Monitoring, cap color is activated Bitte TEXT prüfen |  |  |  |
| Step 1 | Note the value \Monitoring, cap color\ Monitoring, cap color \Consecutive fault: _____ Bitte Pfad /TEXT prüfen Set “Consecutive fault” to 3 and transmit setting to the control system Manipulate the colour of 3 vials. Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after the labels with the unreadable code have passed the code scanner |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault: Bottle cap color |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set “Consecutive fault” to the value noted in step 1 and transmit parameter to control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |