| Description | OQ Alarm: MX_170_MS_SfMonitoring |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Consecutive fault: Carton labeling |  |  |  |  |
| Prerequisites | Machine ready for operation and filled with products \Monitoring, carton labeling\Settings “Monitoring, carton labeling” is activated Bitte TEXT prüfen \Carton imprint\Settings “Printer” on HMI is set to Auto Bitte TEXT prüfen |  |  |  |  |
| Step 1 | Note the value \Monitoring, carton labeling\Monitoring, carton labeling “Consecutive fault”: ___ Bitte Pfad /TEXT prüfen Set “Consecutive fault” to 3 Manipulate the labeling area of three cartons so the imprint cannot be verified. Place the cartons in front of the carton magazine Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | The machine stops after the manipulated cartons have passed the camera. |  |  |  |  |
|  | The following message appears: |  |  |  |  |
|  | Consecutive fault: Carton labeling |  |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |  |
|  | The machine cannot be started. |  |  |  |  |  |
| Step 3 | Set “Consecutive fault” to the value noted in step 1 and transmit setting to the control system Bitte TEXT prüfen Press the RESET-button |  |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |  |
|  | Message is cleared. |  |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |  |