| Description | OQ Alarm: MX_202_MS_CFLaser |  |  |  |
| --- | --- | --- | --- | --- |
|  | Serienfehler: Laserkennzeichnung |  |  |  |
| Prerequisites | Machine is ready for operation and filled with product Operation\Laser labeling\Laser is switched on check path |  |  |  |
| Step 1 | Note value \Laser labeling\Laser labeling “Consecutive fault”: _____ Set “Consecutive fault” to 1 Disconnect input .004-K20.20:LED11 check BMK Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after the first tube has passed the Laser head |  |  |  |
|  | The following message appears: |  |  |  |
|  | Serienfehler: Laserkennzeichnung |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set “Consecutive fault” to the value noted in step 1 Reconnect input Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |