| Description | OQ Alarm: MX_2505_MF_UprightProduct |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Upright product in the product chain. Please remove product from product chain and then confirm removal here. |  |  |  |  |
| Prerequisites | Machine ready for operation Put one vial upright in the product chain between star wheel transfer and sensor .2505-B3. Bitte BMK prüfen |  |  |  |  |
| Step 1 | Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | Machine stops, when the sensor detect a product |  |  |  |  |
|  | The following message appears: |  |  |  |  |
|  | Upright product in the product chain. Please remove product from product chain and then confirm removal here. |  |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started |  |  |  |  |
| Step 3 | Remove upright product Press the RESET-button at the dialog message in the HMI |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |