| Description | OQ Alarms: MX_500_MS_FlapOverload |  |  |  |
| --- | --- | --- | --- | --- |
|  | Overload flap transfer |  |  |  |
| Prerequisites | Machine ready for operation |  |  |  |
| Step 1 | Open protective cover Manually open flap Close protective cover |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Following message appears: |  |  |  |
|  | Overload flap transfer |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |