| Description | OQ Alarm: MX_501_MS_SfProdMissing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Consecutive fault: Product not transferred |  |  |  |  |
| Prerequisites | - Machine is ready for operation and filled with product - Bottle is set to automatic |  |  |  |  |
| Step 1 | Note the value \Operation\Auger transfer\Auger\Consecutive fault: ___ Bitte Pfad /TEXT prüfen Set “Consecutive fault” to 3 Transmit settings to the control system Manipulate inserting wheel transfer so that no product can be drawn off Press the START button |  |  |  |  |
| Result 1 | Expected |  | Pass / Fail: Dev-No | Date | Initials |
|  | - Machine stops |  |  |  |  |
|  | - The following message appears |  |  |  |  |
|  | Consecutive fault: Product not transferred |  |  |  |  |
| Step 2 | - Press the START button |  |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No |  | Date | Initials |
|  | - The machine cannot be started |  |  |  |  |
| Step 3 | - Set „Consecutive fault“ to the value noted in step 1 - Transmit settings to the control system - Press the RESET button |  |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No |  | Date | Initials |
|  | - Message is cleared. |  |  |  |  |
|  | - Machine status: „Ready for operation“ |  |  |  |  |