| Description | OQ Alarm: MX_505_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault, add-on transfer |  |  |  |
| Prerequisites | Machine is ready for operation Activate „Booklet automatic“ on the HMI Deactivate all other monitoring |  |  |  |
| Step 1 | Note consecutive fault ___ (Settings/Feed in add on/Inserter (booklet)) check Path Set consecutive fault to 3 and send setting to control Press the START-button After a few packages were formed press the STOP button. Remove three consecutive booklets before they reach the control sensor Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after 3 not detected booklets |  |  |  |
|  | The following message appears: |  |  |  |
|  | Consecutive fault, add-on transfer |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Press the RESET-button Set consecutive fault to value noted in step 1 and send setting to control |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |