| Description | OQ Alarm: MX_506_MS_ConsFaultChain |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Consecutive fault, Leaflet not present in leaflet clip chain |  |  |  |  |
| Prerequisites | Machine ready for operation. \Leaflet\ Settings\Leaflet is set to Auto Bitte Text prüfen |  |  |  |  |
| Step 1 | Note the value of \Leaflet\Leaflet transfer\Consecutive fault, leaflet missing in leaflet clip chain___ Bitte Text prüfen Set “Consecutive fault, leaflet missing in leaflet clip chain” to 3 and send setting to the control system Press the START-button and wait until the first products have been detected by the product sensing. Press the STOP-button, open protective guarding and remove 3 consecutively following leaflets out of the leaflet chain between the leaflet unit and the product chain. (if it is necessary to get 3 leaflets start and stop the machine again) Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | Machine stops after three not detected leaflets. |  |  |  |  |
|  | The following message appears: |  |  |  |  |
|  | Consecutive fault, Leaflet not present in leaflet clip chain |  |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |  |
| Step 3 | Set “Consecutive fault, leaflet missing in leaflet clip chain” to the noted value Transmit setting to the control system Press the RESET-button |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |