| Description | OQ Alarm: MX_506_MS_ConsFaultPocket |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Consecutive fault, Leaflet not drawn off in leaflet unit |  |  |  |  |
| Prerequisites | Machine ready for operation and filled with product. \Leaflet\ Settings\Leaflet” is set to “Auto” Bitte Text prüfen |  |  |  |  |
| Step 1 | Note the value of \Leaflet\Leaflet transfer\Consecutive fault, leaflet not drawn off: ___ Bitte Text prüfen Set “Consecutive fault, leaflet not drawn off” to 3 and send transmit setting to the control system Remove leaflets from leaflet feeder magazine Cover sensor .506-B3 and .509-B1 Bitte BMK Prüfen, .509-B1 only if with prewarning Press the RESET-button Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | Machine stops after three not retrieved leaflets. |  |  |  |  |
|  | The following message appears (additional messages may be displayed additionally): |  |  |  |  |
|  | Consecutive fault, Leaflet not drawn off in leaflet unit |  |  |  |  |
| Step 2 | Press the START-button. |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |  |
| Step 3 | Uncover sensors Put the leaflet stack onto the leaflet feeder magazine. Set “Consecutive fault, leaflet not drawn off” to the noted value and transmit setting to the control system Press the RESET-button |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |