| Description | OQ Alarm: MX_506_MS_Overload Sensor .506-B6 ist der Referenzsensor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Overload clutch at leaflet unit has been tripped |  |  |  |  |
| Prerequisites | Machine is ready for operation \Leaflet\Settings “Leaflet” is set to Auto Bitte Text prüfen |  |  |  |  |
| Step 1 | Disconnect sensor .506-B6 overload leaflet unit at input .004-K20.3:LED3 Bitte BMK / input Prüfen Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | Machine starts and stops immediately |  |  |  |  |
|  | The following message appears: |  |  |  |  |
|  | Overload clutch at leaflet unit has been tripped |  |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |  |
| Step 3 | Reconnect input Press the RESET-button |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |