| Description | OQ Alarm: MX_536_MF_ProductUpright |  |  |  |
| --- | --- | --- | --- | --- |
|  | Upright product |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Insert an upright product into the product chain ahead of sensor .536-B1 Check BMK Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine stops |  |  |  |
|  | The following message appears: |  |  |  |
|  | Upright product |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Remove the product Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |