| Description | OQ Alarm: MX_532_MS_CfProdSense |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Consecutive fault: product sensing |  |  |  |  |
| Prerequisites | Machine ready for operation. Select “Trays automatic” on the HMI. \Cycloid\Product sensing. Note serial fault___ Set serial fault to 3 and send setting to control |  |  |  |  |
| Step 1 | Remove trays from cycloid transfer Cover sensor .532-B5 Bitte BMK Prüfen Press RESET-button Press the START-button |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | Machine stops |  |  |  |  |
|  | The following message appears: |  |  |  |  |
|  | Consecutive fault: product sensing |  |  |  |  |
| Step 2 | Press the START-button |  |  |  |  |
| Result 2 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |  |
| Step 3 | Take cover from sensor Put the trays back to the magazine. Set serial fault to noted value and send setting to control Press the RESET-button |  |  |  |  |
| Result 3 | Expected |  | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |  |