| Description | OQ Alarm: VC_MX_700_MS_UprightProd |  |  |  |
| --- | --- | --- | --- | --- |
|  | Upright product in the pick-up area of the robot |  |  |  |
| Prerequisites | Machine is ready for operation “Combined operation” is set to On Bitte TEXT prüfen |  |  |  |
| Step 1 | Disconnect sensor .700-B1 CHECK BMK Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The following message appears: |  |  |  |
|  | Upright product in the pick-up area of the robot |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Reconnect sensor Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status: ”Ready for operation” |  |  |  |