| Description | OQ Alarm: MX_270_MS_NozzleUp |  |  |  |
| --- | --- | --- | --- | --- |
|  | Dust extraction nozzle is not in home position |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Disconnect cable from sensor .270-B1 |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The following message appears: |  |  |  |
|  | Dust extraction nozzle is not in home position |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Reconnect cable from sensor .270-B1 Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |