| Description | OQ Alarm: MX_043_MS_BadNotDisch |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bad carton not ejected |  |  |  |
| Prerequisites | - Machine is ready for operation. |  |  |  |
| Step 1 | - Pull the air hose from suction arm at bad carton ejection. - Simulation Produce bad one carton of the function before carton ejection station. - Press START button. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The suction arm cannot remove the carton and the machine stops |  |  |  |
|  | - Dialog box show up: - Remove package from cartonchain at ejection station - Press the RESET button dialog box |  |  |  |
| Step 2 | - Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - Remove the carton at bad carton discharge. - Install the air hose back. - Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The message disappears. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |