| Description | OQ Alarm: MX_049_MS_GoodNotDisch |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Good carton not discharged |  |  |  |  |
| Prerequisites | - Machine is ready for operation. |  |  |  |  |
| Step 1 | - Press the START-button. - Produce good carton and running the machine until discharge station. - Press the STOP-button and cover senser .049-B5 - Press the START-button again. |  |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. |  | Date | Initials |
|  | - The machine stops immediately. - Dialog show up. - Remove package from cartonchain at ejection station |  |  |  |  |
| Step 2 | - Press Reset button in the dialog box. |  |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date |  | Initials |
|  | - Dialog box disapears. - The following messages - appears: Good carton not discharged |  |  |  |  |
| Step 3 | - Take out the carton from the carton chain. - Uncover senser .049-B5 - Press the RESET button. |  |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date |  | Initials |
|  | - The message disappears. |  |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |  |