| Description | OQ Alarm: MX_103_MS_CfDustFlapBot |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault dust flap bottom |  |  |  |
| Prerequisites | Machine is ready for operation and filled with products Activate \Monitorin dust flap\Settings “Dust flap bottom” check text Register value of \Monitoring dust flap\Monitoring dust flaps “Consecutive fault dust flap monitoring”: ___ check text and set it to „3“ Send setting to the control system |  |  |  |
| Step 1 | Manipulate the trailing frontal dust flap of 3 cartons, so that sensor .103-B1 check CIS can detect the flap. Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | After the 3 manipulated cartons have passed the sensor, the machine stops |  |  |  |
|  | Following message appears: |  |  |  |
|  | Consecutive fault dust flap bottom |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. |  |  |  |
| Step 3 | Set Consecutive fault(s) to noted value and transmit setting to the control system Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |