| Description | OQ Alarm: VC_MX_522_MF_ErrRemoveProd |  |  |  |
| --- | --- | --- | --- | --- |
|  | Remove products from transfer |  |  |  |
| Prerequisites | - Machine is ready for operation in production mode. |  |  |  |
| Step 1 | - Set the Transfer Swivel Lever / Automatic to ON. - Remove product from transfer system (manipulate of the running the machine without product). - Cover a sensor .300-B6 - Cover a sensor .300-B2 - Press the START-button. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The transfer starts the motion to feed the product into the carton. - The machine stops in home position. |  |  |  |
|  | - The machine stops - The following message appears: - Remove products from transfer |  |  |  |
| Step 2 | - Uncover a sensor. - Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - Press Reset button dialog box show up |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |