| Description | OQ Alarm: MX_522_MF_TransferOverload |  |  |  |
| --- | --- | --- | --- | --- |
|  | Overload product transfer |  |  |  |
| Prerequisites | - Machine is ready for operation. |  |  |  |
| Step 1 | - Disconnect a sensor .522-B5 |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Dialog box show up: Overload product transfer - Press the Reset button dialog box |  |  |  |
| Step 2 | - Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. |  |  |  |
| Step 3 | - Connect a sensor back. - Press the RESET-button |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The message disappears. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |