| Description | OQ Alarm: VI:.VC_MX_040_MS_InvalidBitCombo |  |  |  |
| --- | --- | --- | --- | --- |
|  | Invalid HMI bit combination for coded protective guarding |  |  |  |
| Prerequisites | Machine is ready for operation. |  |  |  |
| Step 1 | Select Carton erection / Coded protective guarding. Simulate the fault condition by deactivating the "Coding, Position 2 is required". Press Transmit button. press “Transmit“ button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The following message appears: |  |  |  |
|  | Invalid HMI bit combination for coded protective guard-ing |  |  |  |
| Step 2 | Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. RED indicator lamp is “ON”. |  |  |  |
| Step 3 | Set "Coding, Position 2 is required" to the state noted in Step 1 and transmit the parameter to the control system. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |