| Description | OQ Alarm: VI:.VC_MX_043_MF_BadNotEject |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bad carton not completely ejected. Please remove affected carton. Then confirm removal here. |  |  |  |
| Prerequisites | - Machine is ready for operation. |  |  |  |
| Step 1 | - Remove the finger giude at bad carton ejection from its position. - Simulation produces bad one carton of the function before carton ejection station. - Press START button. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The finger giude cannot remove the carton and the machine stops |  |  |  |
|  | - The following message Bad carton not completely ejected. Please remove affected carton. Then confirm removal here. |  |  |  |
| Step 2 | - Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine - cannot be started. - RED indicator lamp is “ON” and the hooter is activated. |  |  |  |
| Step 3 | - Remove the carton at bad carton discharge. - Install the finger guide back. - Press the RESET button. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |