| Description | OQ Alarm: VI:.VC_MX_101_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Carton sensing bottom |  |  |  |
| Prerequisites | Machine is ready for operation in production mode. The machine is filled with products. |  |  |  |
| Step 1 | Set the Carton erectoion / Settings / Cartons /Switch Auto Note the value for Carton erection system / Consecutive fault carton pick up ___. Press Transmit button. Pull the air hose from the suction arm at carton magazine. Press the START-button. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine starts. Cartons are not fed in. Cartons are not recognized by a sensor .101-B1. Check CIS Machine stops in home run position when the number of consecutive fault is reached. |  |  |  |
|  | The following message appears: Consecutive fault: Carton sensing bottom |  |  |  |
| Step 2 | Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine - cannot be started. RED indicator lamp is “ON” and the hooter is activated. |  |  |  |
| Step 3 | Install the air hose at the suction arm at the carton infeed. Set “Consecutive fault” to the value noted in step 1 and transmit parameter to the control system Press the RESET button. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |