| Description | OQ Alarm: Fault: VC_MX_154_MS_ConsFault |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consecutive fault: Booklet code |  |  |  |
| Prerequisites | Machine is ready for operation in production mode. - The machine is filled with products. |  |  |  |
| Step 1 | Set the Monitoring, booklet code / Settings / Monitoring, booklet code is switched ON. Note the value for Monitoring, booklet code/ Consecutive fault _3_. Simulation test by changing the Consecutive Fault setting parameter to __ Press Transmit button. - Insert three Booklets with covered camera/print marks (simulate as if print mark of the cartons defective / faulty) - Press the START-button |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Machine stops after the three manipulated cartons have passed the code reader. |  |  |  |
|  | Following message appears: - Consecutive fault: Booklet code |  |  |  |
| Step 2 | - Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine - cannot be started. - RED indicator lamp is “ON” and the hooter is activated. |  |  |  |
| Step 3 | Set the 'Consecutive Fault' parameter to the value note in Step 1 and Press transmits the button parameter to the control system. Press the RESET button. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The message disappears and the machine is ready for operation. |  |  |  |