| Description | OQ Alarm: VI:.VC_MX_455_MS_NotRighty |  |  |  |
| --- | --- | --- | --- | --- |
|  | Robot not in Righty posture |  |  |  |
| Prerequisites | - Machine is ready for operation. |  |  |  |
| Step 1 | - Open protective guarding at the robot position. - Manually move the robot arm to the left side to simulate an incorrect robot position. - Close protective guarding at the robot position. - Check that the HMI displays |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Dialog box show up: - Robot not in Righty posture. Please manually move the robot back into the Righty posture. - Press the RESET button dialog box |  |  |  |
| Step 2 | - Press the START-button. |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - The machine cannot be started. - RED indicator lamp is “ON”. |  |  |  |
| Step 3 | - Open protective guarding at the robot position. - Manually move the robot arm back to the Righty posture. - Close protective guarding at the robot position. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | - Message is cleared. |  |  |  |
|  | - Machine status operation: ”Ready for automatic drive mode” |  |  |  |