| Description | OQ Alarm: VI:.VC_MX_455_MS_PosLimit |  |  |  |
| --- | --- | --- | --- | --- |
|  | Robot is outside the work area |  |  |  |
| Prerequisites | Machine is ready for operation |  |  |  |
| Step 1 | Select Robot / Settings / Release brake set to ON. Open protective guarding No. 8 while performed alarm. Press “Brake release” button on robot. Move the robot manually outside the defined work area. Close protective guarding No. 8 Check that error message is displayed on the HMI. |  |  |  |
| Result 1 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Typographical error. The following message appears: |  |  |  |
|  | Robot is outside the work area |  |  |  |
| Step 2 | Press the START-button |  |  |  |
| Result 2 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | The machine cannot be started. RED indicator lamp is “ON”. |  |  |  |
| Step 3 | Select Robot / Settings / Release brake set to ON. Simulate robot position in working by mentioned above procedure. Open protective guarding No. 8 Press “Brake release” button on robot. Move the robot manually inside the defined work area. Check that no error message on the HMI Press the RESET-button. |  |  |  |
| Result 3 | Expected | Pass / Fail: Dev-No. | Date | Initials |
|  | Message is cleared. |  |  |  |
|  | Machine status operation: ”Ready for automatic drive mode” |  |  |  |